# Tim Olson  –  Z1721379  –  Proj 3

# Answer these questions in no more than 100 words each

- Draw a classical control loop to describe the robot operation. Clearly label the controller, plant, and sensor and how the desired output is compared with the current output.

The below control loop includes both turning to face the bat, and driving to within a specified distance from the bat. The distance control is disabled until the robot has had 5 seconds to find and face the bat.

+

+

Actuator:

Motors

+

-

Goal:

Bat centered

Plant:

Motor Signals

Controller:

PID

Sensor:

Camera

Bat Centroid X

Goal:

Bat distance ~1ft

+

-

Plant:

Motor Signals

Controller:

PID

Sensor:

Camera

Bat pixel width

- Using the format below to write the pseudocode for Task 1. Even though your team may have the same program, pseudocode is your interpretation of the general approach (or a first stab at it); so this should be your own version.

| Task 1 | Approach an object in the field of view |
| --- | --- |
| Input: | Image and time information |
| Output: | Remove background |
|  | Filter by color |
|  | Find centroid and width of object |
|  | Run turning PID turnSpeed = PID value |
|  | If elapsed time > 5 sec |
|  | Run driving PID |
|  | driveSpeed = PID value |
|  | leftSpeed = distspeed – turnspeed rightSpeed = distspeed + turnspeed |
|  | Output speeds to wheels |
|  | If distance within ~1 ft Disable driving/distance PID |

- Using the format below to write the pseudocode for Task 2. Even though your team may have the same program, pseudocode is your interpretation of the general approach (or a first stab at it); so this should be your own version.

| Task 2 | Follow an object in the field of view |
| --- | --- |
| Input: | Image and time information |
| Output: | Use same code as Task 1 |

- Within your team, who contributed the most to your project? What will you do next time to contribute more?

Myself and Matt contributed the most to this project. Gunnar created a filtering method that Matt developed into image processing to return x-centroid and width of the detected bat. Tom documented a Python tutorial that was presented to several class-mates. I wrote a PID library and the bat facing/following code.